Laser Supply

Voltage	3.3 V

OVP/OCP	3.5 V / 0.03 A

Hamamatsu AMP

RF Range	M or H

Oscilloscope (8-bit)

Coupling / Probe 	DC / 1X

Invert 			ON

Acquire Mode 	Average (16/64) if the jagged line (press the acquire button and change the mode to average (64)

Measure -> Mean

CH1 -> type -> mean

Voltage div	1 V/div or 500 mV/Div

Time div	10 ms/div 	or 50 ms/div

Trigger Level 	auto

Phase 2

Step 1: Dark Baseline (Vdark)

This step removes the electronic noise and ambient light from the data.

- Turn the laser OFF.

- Cover all ports of integrating sphere.

- Adjust the AMP to zero margin to bring the oscilloscope line to 0 V.

- Record the Mean Voltage Vdark

Step 2: Reflectance Calibration (Vstd)

- Configure the sphere for Back Scattering. (Laser at 180° and NIS-PTFE-030 at 0°)

- Turn the laser ON and stabilize the laser for 2 minutes.

- Record the Mean Voltage Vstd.

Step 3: Sample Reflection (Vsample_R)

- Keep the laser at 180°.

- Replace the white plate with phantom at the 0° port.

- Ensure the sample is flush against the port.

- Record the Mean voltage Vsample_R.

Step 4: Transmittance Calibration (V100)

This establishes the “100% Transmission reference”.

- Configure for Forward Scattering: Laser at 0°, 180° port is empty (Open to a dark room or block with a back “light trap”.

- Ensure no sample in the 0° port

- Record the Mean voltage V1 00.

Step 5: Sample Treansmittance (Vsample_T)

- Keep the laser at 0°.

- Place the Bone Phantom between the laser and 0°.

- Record the Mean Voltage Vsample_T.

Ensure all the laser and sample are perpendicular.