Fluid-Structure Interaction Investigation of Mechanical Stent Failure in Canine Trachea Subjected to Obstructed Flow

MR. THU TA ZAW

A THESIS SUBMITTED IN PARTIAL FULFILLMENT

OF THE REQUIREMENTS FOR

THE DEGREE OF MASTER OF ENGINEERING

(MECHANICAL ENGINEERING)

FACULTY OF ENGINEERING

KING MONGKUT’S UNIVERSITY OF TECHNOLOGY THONBURI

2025

Fluid-Structure Interaction Investigation of Mechanical Stent Failure in Canine Trachea Subjected to Obstructed Flow

Mr. Thu Ta Zaw B.Eng. (Mechatronics and Autonomous Systems)

A Thesis Submitted in Partial Fulfillment

of the Requirements for

the Degree of Master of Engineering (Mechanical Engineering)

Faculty of Engineering

King Mongkut’s University of Technology Thonburi

2025

Thesis Committee

…………………………………………………..	Chairman of Thesis Committee

(                                                    )

…………………………………………………..	Member and Thesis Advisor

(Assoc. Prof. Anak Khantachawana, Ph.D.)

…………………………………………………..	Member

(Assoc. Prof. Wishsanuruk Wechsatol, Ph.D.)

…………………………………………………..	Member

(Assoc. Prof. Anusak Kijtawornrat, Ph.D.)

…………………………………………………..	Member

(Asst. Prof. Kasama Srirussamee, Ph.D.)

Copyright reserved

Thesis Title 	Fluid-Structure Interaction Investigation of Mechanical Stent Failure in Canine Trachea Subjected to Obstructed Flow

Thesis Credits		24

Candidate		MR. Thu Ta Zaw

Thesis Advisor	Assoc. Prof. Anak Khantachawana, Ph.D.

Program		Master of Engineering

Field of Study		Mechanical Engineering

Department		Mechanical Engineering

Faculty 		Engineering

Academic Year	2025

Abstract

Wording………………………………………………………………………………… ………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Keywords:	CFD, FSI, Canine’s Trachea, Tracheal Collapse, Mechanical Stent Failure, Stent Fracture, Stent Migration

ACKNOWLEDGMENTS

Wordings………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Wordings………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Wordings………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Wordings………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Wordings………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

CONTENTS

CHAPTER 1	 INTRODUCTION	1

1.1 Introduction	1

1.2 Objectives	2

1.3 Scopes	3

1.4 Research Questions	3

1.5 Expected Benefits	4

CHAPTER 2	 THEORIES AND LITERATURE REVIEW	5

2.1 Three-Dimensional Model of Stent	5

2.1.1 Geometrical Modeling of Stent	5

2.1.2 Material Constitutive Model of Stent	8

2.2 Three-Dimensional Model of the Canine Trachea	10

2.2.1 Geometrical Modeling of Canine’s Trachea	11

2.3 Fluid-Structure Interaction Analysis	13

2.3.1 Wall Shear Stress (WSS)	15

2.4 Stent Fracture	19

2.4.1 Von Mises Stress	19

2.4.2 Fatigue Criteria	22

2.5 Stent Migration	24

2.5.1 Radial Force	25

2.5.2 Drag Force	26

2.6 Experiments	27

2.5.1 Tensile Testing	27

2.5.2 Mylar Film Testing	28

2.5.3 Particle Image Velocity Experiment	29

CHAPTER 3	 MATERIAL AND METHODOLOGY	30

3.1 Computational Analysis	30

3.1.1 3D Braided Stent Model	30

3.1.2 Canine’s trachea Geometry	32

3.1.3 Fluid-Structure Interaction (FSI) simulations	33

3.1.4 Mesh Convergence Analysis	35

3.1.5 Assumption and simplifications	35

3.2 Experimental Validation	36

3.2.1 Tensile Testing Experiment	36

3.2.2 Mylar Film Testing Experiment	37

CONTENTS (Cont’d)

3.2.3 Particle Image Velocity Experiment	38

3.3 Experimental Schedule	39

# CHAPTER 1	 INTRODUCTION

## 1.1 Introduction

Tracheal Collapse is a chronic respiratory disease found in mature tory and small-breed dogs [1].It is a progressive medical condition caused by the narrowing of the trachea lumen. Several researchers have evaluated the underlying causes behind the development of these progressive diseases.  One of the etiologies of trachea collapse is the reduction of glycosaminoglycan, glycoprotein and chondroitin sulphate content in the hyaline cartilage that forms the cartilage rings of the trachea, which line the tracheal wall. There are four stages of tracheal collapse depending on the percentage of airway obstruction. Clinical characteristics of dogs with tracheal collapse can vary depending on the severity of the tracheal collapse. Some common signs include goose honk, exercise intolerance, cyanosis and potentially life-threatening breathing difficulties that severely impact a dog’s quality of life [2-4].

There are different techniques in diagnosis of the trachea collapse in dogs, including Radiographs, Fluoroscopy, Computed Tomography, Magnetic Resonance Imaging and Ultrasound Imaging. Radiographs are essential to evaluate the lung fields for concurrent respiratory diseases, while fluoroscopy and ultrasound offers dynamic and real-time evaluation of the trachea during all phases of respirations as well as during episodes of coughing. Since Computed Tomography scan provides detailed, three-dimensional images of the trachea, which allows the veterinarians to choose the customize stents that fit accurately [3, 5-8].

Common treatment options for Tracheal Collapse include medical management and surgical intervention, involving extraluminal ring prostheses or endoluminal tracheal stents, depending on the severity of the collapse. Medical management centers on efforts to minimize the clinical signs and controls the airway inflammation. While extraluminal rings are primarily used in the cervical trachea, endoluminal stents offer the advantage of treating the entire trachea with minimal downtime. Even though endoluminal tracheal stenting comes with several merits, the complications related to this operation such as stent migration and stent fracture should be considered. Minor complications may require medical management, whereas major complications may require a second stenting procedure [9-13].

To mitigate the complications related to the stent, recent research has focused on optimizing the design and material properties of tracheal stents to enhance overall performance, ensuring they provide the sufficient radial force to maintain airway patency without causing excessive pressure on the tracheal lumen, which could lead to tissue damage and restenosis [13]. However, performing the experiment multiple times in the laboratory is expensive and time-consuming, so researchers use computer simulations to reduce the cost. Numerous computational models have emerged in recent years for analysis of endovascular stents, such as computational fluid dynamic and finite element analysis [14-36]. One of the most promising techniques is Fluid-Structure Interaction (FSI) analysis, which simulates the interaction between the fluid mechanics and structural mechanics of the stent and tracheal wall. FSI simulations allow researchers to predict effectiveness and how a stent will behave when deployed into the trachea [14, 19, 21, 22, 25, 26].

Recent studies have applied FSI analysis to endovascular stents for the arteries and tracheas of human [14, 19-23, 25-27]. However, computational studies focused specifically on trachea stents for dogs are very limited [16, 17, 36]. In this study, FSI modeling will be performed to analyze the behavior of a stent in the dog with Tracheal Collapse.

## 1.2 Objectives

- To perform FSI simulations to analyze the effect of stent deployment in the obstructed canine trachea and to evaluate the potential of restenosis.

- To conduct FSI simulations on a canine tracheal model after stent deployment in the stenotic region to identify potential causes of mechanical stent failure.

- To validate simulation results through experiments.

## 1.3 Scopes

- The study is performed through Fluid-Structure Interaction (FSI) simulations, and experiments.

- The creation of the SUPERA stent 3D model is performed using a cylindrical diamond-shaped braiding equation, and the Auricchio model is applied to capture the Nitinol shape memory effect.

- The 3D geometry of the canine’s tracheal model is created based on the CT-scan of Pomeranian dog with tracheal stenosis and model the tracheal wall as a homogenous structure.

- The Fluid-Structure Interaction (FSI) simulations for obstructed trachea and stented trachea are performed by coupling the ANSYS Fluent and ANSYS Static Structural with ANSYS System Coupling in ANSYS work bench.

- The analysis of Wall Shear Stress (WSS) is done in obstructed and stented airways to evaluate the effect of stent inside the canine trachea.

- The analysis of Von Mises Stress, radial force and drag forces are conducted in FSI simulation on the stented idealized model to analyze the potential stent fracture and migration.

- The tensile strength testing of the stent wire is done to validate the nitinol constitutive model.

- The Mylar Film testing experiment is done to validate the stent radial force obtained from FSI simulations.

- Particle Image Velocity experiments are done and compared with the results from the simulation data to validate the flow patterns.

## 1.4 Research Questions

- Can the braided Nitinol stent withstand the compressive forces exerted by the tracheal wall without experiencing structural failure or fracture?

- Is the stent capable of maintaining mechanical integrity over a projected fatigue life of 10 years under cyclic loading induced by chronic coughing?

- Does the stent generate sufficient radial force to ensure stable anchorage within the tracheal lumen, thereby preventing migration or detachment?

- Can the stent endure aerodynamic drag forces resulting from airflow within the trachea without compromising its position or structure?

## 1.5 Expected Benefits

- Interaction of airflow and structural forces with the stent and tracheal wall can be understood.

- More accurate predictions in the improvement of breathing efficiency and reducing the risk of mechanical stent failure can be obtained by comparing rigid and compliant tracheal walls.

- Future investigations into tracheal prosthesis designs in veterinary medicine and human airway disorders can be performed based on this research.

# CHAPTER 2	 THEORIES AND LITERATURE REVIEW

This chapter provides an overview of key theories and research related to Computational Fluid Dynamics (CFD), Fluid-Structure Interaction (FSI), and various computational analyses conducted on stents and canine tracheal collapse. Topics covered include stent geometry and material properties, canine tracheal material properties, as well as CFD and FSI simulations and their parameters of interest. Through this review, relevant methodologies, findings, and gaps in current research are identified to support the scope of this study.

## 2.1 Three-Dimensional Model of Stent

Stents are a type of prosthesis with a small cylindrical structure that is inserted into the affected region via a catheter [37].The primary purpose of a stent is to provide structural support to restore airway’s patency, keep the trachea open, and improve breathing [38, 39]. There are various types of stents available on the market, classified based on their design, material, and clinical usage, such as vascular, nonvascular, coronary, peripheral, and tracheal stents [40].

Since stent modeling considers both material and geometrical parameters, it can reduce the time and cost associated with experiments by simulating deformation, radial force, stresses, and their interaction with the deployment sites [41].

### 2.1.1 Geometrical Modeling of Stent

When determining the mechanical behavior of a braided stent, geometrical modeling plays an essential role. Since there are five basic types of stent geometries (coil, helical spring, woven stents (braided, knitted), rings (individual, sequential), and cell-based designs (closed, open) [42],analytical formulas are used to construct stent geometries in computational research [42].Complex 3D braided geometries are generated using 3D parametric equations, and Finite Element Analysis (FEA) simulations are conducted to compare different contact strategies, as studied by Alissa Zaccaria [42].

# CHAPTER 3	 MATERIAL AND METHODOLOGY

This chapter outlines the methodologies employed in investigating the fluid-structure interaction (FSI) in stented canine’s trachea with tracheal collapse syndrome (TCS) by integrating modeling and experimental validation to assess the mechanical behavior and airflow characteristics associated with tracheal stenting. The process of creating a 3D braided stent model, reconstruction of tracheal geometry from CT-scan of a dog with TCS and the coupled FSI simulation framework, focusing on the interaction between stent with tracheal wall and airflow are presented in this chapter. Additionally, the experimental setup used to validate the simulation results, ensuring the credibility and relevance of the computational predictions are also described.

## 3.1 Computational Analysis

### 3.1.1 3D Braided Stent Model

The details of fabrication process will not be included in this study. The stent prototypes are fabricated using braiding technology, which include the interlacing of wire between the

## 3.3 Experimental Schedule

| No | Task/Deliverable | Month (2024) |  |  |  |  |  | Month (2025) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 8 | 9 | 10 | 11 | 12 | 1 |  | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | Literature Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Investigating FSI techniques |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Stent Design (CAD) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Trachea 3D model from CT-scan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Establishment of study parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Generate Computational Mesh |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | Define Boundary Condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Perform CFD Simulations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Join Double Degree Program in Kanazawa University |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| No | Task/Deliverable | Month (2026) |  |  |  |  |  | Month (2027) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 8 | 9 | 10 | 11 | 12 | 1 |  | 2 | 3 | 4 | 5 |  |  |  |  |  |  |  |
| 9 | Analyze the CFD results |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | Perform FSI Simulations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | Analyze the FSI results |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | Compare the Simulation and Experimental Results |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | Discussion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | Cohesion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Planned Schedule |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Done |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | DDP Program Timeline |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3.4 Experiment Schedule

# CHAPTER 4	 RESULTS

# CHAPTER 5	 CONCLUSION

REFERENCES

1.	Amis, T., Tracheal collapse in the dog. 1974.

2.	Jung,