Viscous Damping Coefficient of a moving object

BY

THU TA ZAW

Measure the viscous damping coefficient of something moving.

Conduct a mini-experiment at the lowest possible cost.

Verify that you actually measured the viscous damping, not friction.

Note: Please show not only the experimental results, but also the idea (how to measure), measurement theory, theoretical resolution, accuracy, cost, evaluation/discussion etc.

In this assignment, I tried to measure the viscous damping coefficient c, of a stack of coin moving vertically in water.

Five one-yen coins were stacked together with tape, which behaved like one rigid cylinder. The 1-L plastic container with transparent side was filled with water. A rule was attached to the side of the container to provide a scale. A smartphone camera with 60 fps was used to record the falling object.

The weight and size of the coins were obtained from the official published documents and used them as a reference.

The stack of coins is assumed as a rigid body moving straight down.

Forces acting on the coins stacked

(free body diagram) coin photo

Gravity: Fg=mg

Buoyancy: Fb=ρfVg (upward)

Viscous Drag: Fd=-cv (upward, proportional to velocity)

ρf = water density

V = volume of coin stack

M = mass of coin stac

V = downward velocity.

Equation of motion:

mdvdt=Fg- Fb-cv=mg- ρfVg -cv

Effective weight:

Weff=m-ρfVg

Then:

mdvdt=Weff-cv

This is a first-order linear ODE. Solve with v0=0:

vt=vt (1-e-tτ)

where:

Terminal velocity:

vt=Weffc

Time constant:

τ=mc

So,

c=Weffvt

| Name | Value |
| --- | --- |
| Weight | 1.0 g |
| Diameter | 20.0 mm |
| Thickness | 1.2 mm |
| Density | 2.7 g/cm |

Table. 1-yen coin’s data

From the experiment:

The coin stack travel 20.5 cm in 1.1s

Distance: y = 0.205 m

Time: t = 1.1 s

I assumed the velocity that calculated from the experiment as an average velocity.

vavg=yt= 0.2051.1=0.186 ms-1

I will assume that the coin stack quickly reaches terminal speed and then moves almost at vt,

vt≈ 0.186 ms-1

Mass: m = 0.005 kg

Volume: V = 1.88 x 10-6 m3

Water density: ρf = 1000 kgm-3

Gravity: g = 9.81 ms-2

Buoyant mass:

mwater=  ρfV=1.88 x 10-3kg

Effective mass:

meff=m-mwater=0.00312 kg

Effective weight:

Weff=meffg=0.00312 x 9.81≈ 0.0306 N

c=Weffvt≈ 0.03060.186≈0.164Nm-1

c≈0.164 Nm-1

τ=mc ≈ 0.0050.164≈ 0.030 s

τ≈ 0.030 s

3τ≈ 0.090 s

By analyzing the result, the coin stack will already arrive >95% of its terminal speed within ~0.1 s.

THU TA ZAW

2415121095